EFFECTIVE SEARCH TECHNIQUE by Tony Wilson

When Searching, it is typical to either wander from a to b (we all do this especially at a rally) and miss ground in front and around you, and finds!

The Wanderer!

This can be rectified though by griding a small section of you field (about 50ft by 50ft or 100ft by 100ft) and covering just that area (from all sides following the direction of the arrows) this can take a long time to cover but will locate everything within the grid .

The Grid!

When the grid is complete, you grid out the next available piece of the field you haven’t covered and keep this going until the field is complete.

A more effective method of covering a field without missing areas is to use a recursive E pattern. You start off at your desired position in the field from any of its sides, you mark out a row of 30ft -40ft and this is the row you will stick to until you meet the other side, when a new row can be started.

The E Pattern!

2

Row 1 30ft wide, the E pattern, this pattern is repeated up the row until you get to the end.

1

3

Direction of your row

For example you have your detector in you right hand and are starting at point 1, walk to the edge of your row, (use a broom handle with a sharpened point to mark the start of your row, as you move up the field you move the stick up with you, this will help you maintain the edge of your row).

Remember not to over swing your detector, the swing should be just past your left foot ( if your working right handed that is, otherwise it is reversed) and 2ft past your right, this will stop you getting to knackered and help keep you coil where it is suppose to be, near the ground.

When you reach the edge (point 2) place your coil 2ft to your right on the ground, walk forward past the centre point and 2ft past where the coil was placed on the ground, walk back to the start edge (point 3), you have just completed a 4ft patch 30ft long and should not have missed any ground.

Now turn and walk back to the opposite edge, following as near the line you have just walked, When you reach the edge, place your coil 2ft to your right on the ground, walk forward past the centre point 2ft past where the coil was placed on the ground, walk back to the start edge, this is the completed E pattern, it now starts again until you get to the end of your row, oh and don’t forget your stick if you used one.

The Lads at Detectorfinds.co.uk